# Why GPS Doesn't Work Underwater

GPS relies on radio waves transmitted from satellites orbiting Earth. In air, these signals travel at the speed of light with minimal loss. However, water absorbs radio waves almost completely — the signal is attenuated by roughly

3.5 dB per metre of seawater. Even at a depth of just 1 metre, the GPS signal is effectively zero.

This means your Plastic Detection Robot cannot use the NEO-M8N GPS module (already in your component list) while submerged. Instead, underwater robots must use one of four alternative approaches described in this document.

QUICK COMPARISON — ALL 4 METHODS

| Method | Principle | Accuracy | Range | Cost (Approx) | Best For Your Robot |
| --- | --- | --- | --- | --- | --- |
| Surface-Dive GPS | Surface for GPS fix, then submerge | 3–5 m | Any | Free (NEO-M8N) | - Start here — free |
| USBL Acoustic | Surface beacon + ROV transponder ping | 1–3 m | 300–1000 m | ~2 L | - Best affordable upgrade |
| DVL Dead Reckoning | Doppler velocity log + compass | 0.1–1% drift/dist | Any depth | ~1–3 L | Good for currents |
| LBL Acoustic | Array of seabed beacons | 10 cm | Site-wide | ~5 L+ | Overkill for rivers |

METHOD 01

Surface-Dive GPS

Robot surfaces periodically to get a GPS fix, then dives back down

The simplest and cheapest approach. Your robot uses the NEO-M8N GPS module already in component list. The robot patrols underwater, and every 1–3 minutes, rises to the surface, locks a GPS coordinate, logs it along with the latest depth/sonar data, then dives again. This is sufficient to pinpoint plastic locations for a cleanup team.

How it works in your robot:

- Robot patrols at depth, scanning with Ping360 sonar + camera

- On detection OR every 2 minutes  autopilot surfaces

- NEO-M8N acquires fix (typically 5–15 seconds cold, 1–3 seconds warm)

- GPS coordinates + timestamp written to onboard storage

- Alert sent via Wi-Fi/4G (if tether or surface buoy present)

- Robot dives back to patrol depth

HARDWARE REQUIRED — From Your XLSX

| Component (from XLSX) | Cost (INR) | Dimensions | Weight | Power | Key Challenges |
| --- | --- | --- | --- | --- | --- |
| NEO-M8N GPS Module (already in your list) | 800 – 1,500 | 25 × 25 mm | 20 g | 0.5 W | Switch between dead-reckoning (underwater) and GNSS (surface) |
| Jetson Xavier NX (AI Compute — item 5.1) | 50,000 – 52,000 | 100 × 87 × 16 mm | 280 g | 10–25 W | Thermal design in sealed enclosure; power stability under bursts |
| Bar30 Depth Sensor (item 3.1) | 6,700 – 7,500 | 24 × 24 mm | 15 g | 0.01 W | Water ingress; temperature drift; I2C bus reliability |
| T200 Thrusters ×4 (item 6.2) | 19,000 – 24,000 (per unit) | 113 mm length | — | 200–500 W | Battery sizing; thermal load on ESCs |
| 24V 20Ah Li-ion Battery (item 7) | Custom build | — | — | — | Thermal runaway; waterproof enclosure; charging compliance |

- CHALLENGES

- Accuracy between surfacings: robot position at depth is unknown — only logged at surface fix points

- Turbulent rivers may push robot off course between fixes

- Surfacing interrupts patrol — detection can't happen while ascending / at surface

- In strong current, robot may drift significantly during 2-min submerged windows

★ RECOMMENDED

PHASE 1

- Use this method immediately. Your NEO-M8N is already in  parts list. Program the

autopilot to surface every 2 minutes (or on detection).

METHOD 02

USBL Acoustic Positioning

Ultra-Short Baseline — a surface beacon triangulates the ROV's exact underwater position

USBL (Ultra-Short Baseline) is the most practical real-time underwater positioning system for small ROVs. A sonar transponder mounted on the robot sends acoustic pings to a surface unit (mounted on a boat, buoy, or the tether). The surface unit measures the angle and time-of-flight of the ping, calculates the robot's position relative to itself, and combines this with its own GPS fix to output a real-world latitude/longitude.

How it works step by step:

- Surface GPS unit (e.g. Cerulean ROV Locator Mk II topside unit) acquires its own GPS fix

- ROV transponder sends an acoustic ping

- Surface unit measures: time of arrival + angle of arrival (bearing + elevation)

- Using speed of sound in water (~1500 m/s), range is calculated

- ROV position = surface GPS + computed offset vector  output as Lat/Lon

- QGroundControl displays real-time ROV position on map

HARDWARE REQUIRED

| Component | Cost (INR) | Dimensions | Weight | Power | Key Challenges |
| --- | --- | --- | --- | --- | --- |
| Cerulean ROV Locator Mk II (ROV transponder) | ~2,00,000 (~$2,395 USD) | Small puck ~Ø60 × 30 mm | ~100 g | ~2–5 W | Sync topside + ROV at start of dive; multipath in shallow confined areas |
| Topside USBL Unit (surface beacon) | Included with above | Handheld unit | ~300 g | ~5 W | Mounting on boat or buoy; must track ROV and have clear water view |
| NEO-M8N GPS Module (for topside unit — item 4.1) | 800 – 1,500 | 25 × 25 mm | 20 g | 0.5 W | Needs good sky view for accurate topside fix; affects final accuracy |
| Jetson Xavier NX (item 5.1 — receives USBL data) | 50,000 – 52,000 | 100 × 87 × 16 mm | 280 g | 10–25 W | Serial/USB integration of USBL data into QGroundControl / ArduSub |
| Bar30 Depth Sensor (item 3.1 — supplements USBL depth) | 6,700 – 7,500 | 24 × 24 mm | 15 g | 0.01 W | Used as depth ground-truth alongside USBL 3D position |
| Sonardyne Micro-Ranger 2 (industrial alternative — item 4.1) | 3,30,000 – 3,50,000 | — | 8–12 kg (surface unit) | 100–25 0 W | SVP changes; multipath; system calibration geometry |

- CHALLENGES

- Sound velocity profile (SVP) changes with temperature/salinity — affects accuracy in deep or stratified water

- Multipath reflections (from river banks, bridge pilings) can distort position

- Surface unit must be held steady or mounted on stable platform with clear underwater line-of-sight

- Range limited by acoustic power — Cerulean Mk II rated to ~300 m depth

- Requires operator presence at surface with the topside unit

★ RECOMMENDED PHASE 2

Cerulean ROV Locator Mk II at ~2 lakh is the most affordable real-time underwater positioning compatible with Blue Robotics hardware. Integrates directly with QGroundControl. Upgrade to this when budget allows.

METHOD 03

DVL Dead Reckoning

Doppler Velocity Log + IMU — tracks movement from last known GPS fix

A DVL (Doppler Velocity Log) measures the robot's precise velocity in X, Y, Z by bouncing acoustic beams off the seabed and measuring the Doppler shift. Starting from a known GPS surface fix, the robot integrates velocity + heading over time to maintain a continuous position track — a method called dead reckoning. This gives real-time position without any surface unit, but error accumulates over time.

How it works in your robot:

- Robot surfaces  NEO-M8N acquires GPS fix  this is the "origin point"

- Robot dives; DVL continuously measures velocity (Vx, Vy, Vz) in body frame

- IMU (e.g. built into Pixhawk) measures heading and rotation

- ArduSub fuses DVL + IMU  continuous Lat/Lon estimate

- Position error grows slowly (~0.1–1% of distance travelled)

- Robot surfaces periodically to reset position error with GPS

HARDWARE REQUIRED

| Component | Cost (INR) | Dimensions | Weight | Power | Key Challenges |
| --- | --- | --- | --- | --- | --- |
| DVL Unit (e.g. WaterLinked DVL A50) | ~3,50,000 (~$4,200 USD) | Ø130 × 57 mm | 640 g | 15 W | Must have clear seabed view (>0.5 m); acoustic interference from T200 thrusters |
| IMU / Flight Controller (Pixhawk — already likely in design) | ~5,000 – 12,000 | 38 × 38 mm | 15 g | 1–2 W | Magnetic interference from thrusters requires careful calibration |
| NEO-M8N GPS Module (item 4.1 — for surface reset) | 800 – 1,500 | 25 × 25 mm | 20 g | 0.5 W | Switching between surface GNSS and underwater dead-reckoning |
| RTK GPS Here3 (item 4.2 — higher accuracy reset) | 25,000 – 30,000 | 78 × 78 mm | 72 g | 1–2 W | Maintaining RTK fix in dynamic environments; antenna EMI issues; needs IMU unit for stability |
| Jetson Xavier NX (item 5.1 — fuses all sensor data) | 50,000 – 52,000 | 100 × 87 × 16 mm | 280 g | 10–25 W | Real-time DVL data fusion pipeline; storage reliability in sealed housing |
| Bar30 Depth Sensor (item 3.1) | 6,700 – 7,500 | 24 × 24 mm | 15 g | 0.01 W | Supplements DVL with accurate depth; I2C bus reliability in EMI environment |

- CHALLENGES

- DVL requires clear seabed returns — won't work in very shallow water (<0.5 m) or if seabed is very soft/muddy

- Position error accumulates: at 0.5% drift, after 100 m travel you have ~0.5 m error

- T200 thrusters create acoustic noise that can interfere with DVL beams — need careful mounting

- Expensive: DVL units start at ~3.5 lakh — significant budget addition

- Also enables station-keeping (hovering in current) — useful bonus for your inspection use case

METHOD 04

LBL Acoustic Positioning

Long Baseline — array of seabed transponders gives survey-grade 10 cm accuracy

LBL (Long Baseline) provides the highest accuracy of all methods — typically 10 cm. Multiple acoustic transponders (usually 3–4) are deployed on the seabed at known surveyed positions. The robot's transponder pings all of them; by measuring the round-trip travel time to each, its position is calculated by trilateration — exactly like GPS, but using sound instead of radio waves.

How it works:

- Deploy 3–4 transponder nodes on the seabed at known GPS-surveyed positions

- Each node is anchored and has its own battery + acoustic modem

- ROV pings all nodes simultaneously

- Range to each node = (travel time × speed of sound) / 2

- Trilateration of 3+ ranges  exact 3D position of ROV

- Accuracy: 10 cm RMS in good conditions

HARDWARE REQUIRED

| Component | Cost (INR) | Dimensions | Weight | Power | Key Challenges |
| --- | --- | --- | --- | --- | --- |
| LBL Transponder Array (e.g. EvoLogics or Sonardyne USBL+LBL) | 5,00,000+ (3–4 node set) | Each ~200 mm cylinder | 2–5 kg each | 5–15 W each | Deployment and recovery logistics; calibration of node positions; acoustic interference |
| ROV Acoustic Transducer (ROV-side node) | Included in set | ~Ø80 × 100 mm | 300 g | 5–10 W | Mounting on ROV frame without blocking thrusters; cable routing |
| Topside LBL Controller (deck unit) | Included in set | Rack/handheld unit | 2–10 kg | 50–100 W | Network bandwidth; real-time processing; laptop + software license required |
| NEO-M8N GPS (item 4.1 — for initial node survey) | 800 – 1,500 | 25 × 25 mm | 20 g | 0.5 W | Must GPS-survey each node position precisely before deployment |
| Jetson Xavier NX (item 5.1 — receives LBL position data) | 50,000 – 52,000 | 100 × 87 × 16 mm | 280 g | 10–25 W | Integration of LBL data with existing ArduSub navigation stack |

- CHALLENGES

- Very complex setup: requires deploying, GPS-surveying, and recovering seabed transponders

- Overkill for river/lake plastic patrol — LBL is designed for oil & gas, offshore survey

- High cost (5 lakh+) and needs trained operators

- Cannot move the survey area easily — nodes must be re-deployed for each new site

- Speed of sound variation (temperature, salinity) must be carefully profiled

✗ NOT RECOMMENDED for

this project

LBL is professional survey-grade equipment designed for fixed offshore sites. For a mobile river/lake plastic patrol robot, the setup complexity and cost far outweigh the benefits. Stick with Method 1 or 2.

Complete Hardware Components - Positioning System

The following components are directly from Hardware_Components_Detail.xlsx, Positioning System. Both standard and industrial-grade options are listed.

  - — NEO-M8N GPS Module (Standard)

| Parameter | Standard (NEO-M8N) | Industrial (Sonardyne Micro-Ranger 2 USBL) |
| --- | --- | --- |
| Cost | 800 – 1,500 | 3,30,000 – 3,50,000 |
| Dimensions | 25 × 25 mm | — |
| Weight | 20 g | 8–12 kg (surface unit) |
| Power Consumption | 0.5 W | 100–250 W |
| Key Challenges | Switching between underwater dead-reckoning and surface GNSS | SVP changes; multipath and reflections; system calibration and mounting geometry |
| Link / Source | robokits.co.in — ublox-neo-m8n-gps-module | sonardyne.com/product/ranger-2-usbl |

  - — RTK GPS Here3 (Standard)

| Parameter | Standard (RTK GPS Here3) | Note |
| --- | --- | --- |
| Cost | 25,000 – 30,000 | Higher accuracy than NEO-M8N |
| Dimensions | 78 × 78 mm |  |
| Weight | 72 g |  |
| Power Consumption | 1–2 W |  |
| Key Challenges | Maintaining RTK fix in dynamic environments; antenna placement and EMI | Must set up IMU unit alongside to ensure stability and fast positioning detection |
| Link / Source | robu.in — hex-here4-multiband-rtk-gnss-gps-module |  |

SUMMARY & RECOMMENDATION

Which Method to Use for Plastic Detection Robot

| Phase 1 — Now (0 extra) | Surface-Dive GPS |
| --- | --- |
|  | Use the NEO-M8N already in component list. Program ArduSub to surface every 2 minutes (configurable). GPS fix logged with depth + sonar data. Sufficient for river patrols. Plastic location accuracy: ~3–5 m (good enough for cleanup team). |

| Phase 2 — When budget allows (~2 lakh) | USBL Acoustic (Cerulean Mk II) |
| --- | --- |
|  | Add the Cerulean ROV Locator Mk II USBL system. Real-time Lat/Lon on QGroundControl map while robot is submerged. Compatible with Blue Robotics hardware and ArduSub/QGC. Depth rating 300 m — far beyond river requirements. Best balance of accuracy, cost, and ease of integration. |

| Phase 3 — Advanced (~3.5 lakh+) | DVL Dead Reckoning (WaterLinked A50) |
| --- | --- |
|  | Add DVL for true autonomous underwater navigation. Station-keeping (hover in current) becomes possible — great for inspection. Fuses with USBL for best-in-class accuracy. Suitable if robot evolves into fully autonomous AUV mode. |

| Not Recommended | LBL Acoustic |
| --- | --- |
|  | Too complex, too expensive, and designed for fixed offshore sites. Not suitable for mobile river/lake patrol missions. |

Note: All dimensions, weights, and power values are approximate and sourced from websites. Prices are indicative and subject to market variation.it should be checked by an expert.

Project: Plastic Detection Robot — Powered by Sirius Software .